| Name: ___________________________________ | Date: ______________ |
| --- | --- |

Cutting Torch

|  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  | 6 |  |  |
|  |  |  |  |  | 7 |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |
|  |  | 9 |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
|  |  |  | 11 |  |  |  |  | 12 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 14 | 15 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 17 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 18 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Across 9. Acetylene cylinder cap- protects the acetylene valve and is left handed 10. Oxygen safety valve- is the valve that is on the top of the oxygen bottle that keeps the oxygen in the 11. Cutting oxygen lever- while cutting this lever is pushed all the way down to help push the flame trough the metal to be sure that the metal is being 12. Acetylene regulator- regulates the pressure that is in the hose and lets you know how much acetylene is in the 13. Oxygen cylinder cap- goes on the oxygen bottle and protects the valve form being damaged. Its right handed 14. Acetylene cylinder valve- where you turn the acetylene on and 17. Oxygen hose- green in color attached to the oxygen 19. Cylinder support- supports the bottles to keep them from moving or falling that would cause an | Down 1. Cutting attachment head- is two chambers that that the gas and oxygen move throw to get to the tip of the 2. Torch oxygen valve- is where the oxygen enters the 3. Oxygen cylinder- the bottle that holds the 4. Acetylene cylinder- the bottle that holds the acetylene. This bottle will be smaller than the oxygen 5. Torch Acetylene valve- where the fuel enters the torch it is left handed 6. Oxygen regulator- regulates how much oxygen presser in the hose and the 7. Acetylene hose- is red in color attached to the acetylene 8. Oxygen cylinder valve- is where you turn on the cylinder and the oxygen comes out of the 15. Cutting tip-is where the oxygen and the fuel mix to cause a reaction that results in a 16. Torch body- this is where the oxygen and the fuel connect to the 18. Secondary oxygen lever- is the lever at the top of the torch where the cutting attachment goes in helps to control the oxygen flow to the cutting |
| --- | --- |