| Name: ___________________________________ | Date: ______________ |
| --- | --- |

Transformer Oil tests

| 1. WHat group of test tells us about the aging of the transformer | A. Karl Fischer |
| --- | --- |
| 2. What test looks for moisture | B. Liquid Power Factor |
| 3. What tst is run at 25 and 100 Degree C | C. D1816 |
| 4. Looking for Incipeant faults | D. Inhibitor |
| 5. What test tells us how much reliable life is left in the solid insulation | E. Dissolved gas analysis |
| 6. looking for dissolved metals | F. Visual Inspection |
| 7. What dielectric test has curved disc electrodes | G. Liquid screen test |
| 8. What test looks for 2.6 Ditertiary Butyl Para-Creesol | H. Furan |
| 9. what test determines the interface between oil and water | I. Interfacial tension (IFT) |
| 10. What test uses Potassium Hydroxide to neutralize the acids in a gram of oil | J. Inductivly coupled plasma |
| 11. What test looks for suspended material, Loudiness, Sediment and turbidity | K. Acid |